November, 2021

Instruction for Roll Damping Prediction Program (Draft)

Osaka Prefecture University

Graduate school of Engineering

Marine system Engineering

0.  INTRODUCTION

This program written in FORTRAN77 calculates the viscous component in roll damping.  The method used as the basis is a component discrete type roll damping prediction method developed in Osaka Prefecture University.  Here, simplified description of the component discrete type roll damping prediction method and description of an input part are shown.  In addition, the notation currently used in the program is indicated with a program.

1.  CHARACTERISTICS OF ROLL DAMPING

In the ship motion in waves, most of hydrodynamic forces acting on hull can be calculated using a potential theory.  However, a roll damping is affected the viscous effect significantly.  Therefore, a calculated result by a potential theory overestimates the roll amplitude in resonance and it is not practical.  So, it is common for a calculation of roll damping to use measured values or estimation method in consideration of a viscosity.

The roll damping prediction method used here is a component discrete type prediction method which was developed by Ikeda et al and which was developed for general-cargo type of ships (round bilge vessel).  The roll damping is consisted with five components which are friction component, wave making component, eddy making component, lift component and bilge-keel component.  It has been confirmed that estimated results for general-cargo type of ships have adequate accuracy.  In the following section, each component is briefly explained with references.

1-1)  Frictional Component

Ref 1)	Hiroshi Kato: On the Frictional Resistance to the Rolling of Ships, Journal of the Society of Naval Archiect of Japan, No.102, Nov. 1957, pp.115-122 (in Japanese).

Ref 2)	Shin Tamiya and Takashi Komura: Topics on Ship Rolling Characteristics with Advance Speed, Journal of the Society of Naval Architects of Japan, No.132, Oct. 1972, pp.159-168 (in Japanese).

Frictional component takes 8 to 10% of a total roll damping for about 2m in overall-length model ship.  This component has Reynolds effect (scale effects), and the rate decreases in proportion to ship size and it takes about 1 to 3% for real scaled ships.  Other components of a roll damping do not have a scale effect.  Therefore, even if the scale of a ship is changed, the non-dimensional damping coefficient can be used as the same value.  For estimation of this component, the following equation which is the modified Kato’s equation at Fn=0 by Tamiya’s advanced-speed modification is used.

BF=43πρ Sf rf3 φ0 ω Cf1+4.1UωL

In the equation, Cf (; the equivalent flat plate frictional coefficient), rf (the equivalent radius), Sf (the surface area) are denote the following equations.

Cf=1.3283.22rf2 φ02T ν−0.5

rf=0.887+0.145 Cb1.7d+Cb B−2OGπ

Sf=L 1.75d+CbB

Where ν is the coefficient of kinematic viscosity, T is roll period, OG is distance from the still water surface to the center of gravity (down ward +).

1-2)  Wave Making Component

Ref 3)	Yoshiho Ikeda, Yoji Himeno and Norio Tanaka: Component of Roll Damping of Ship at Forward Speed, Journal of the Society of Naval Architects of Japan, No.143, May 1978, pp.121-133 (in Japanese).   (+ in English)

This component takes about 5 to 30% for general-cargo type ships.  However, the component may account big rate for a kind of ships with shallow draught and wide section.  The component at Fn=0 can be calculated by a linear potential theory based on small wave height.  The effects of forward speed the component is taken into consideration using the following modification equation.

BwB=0.5A2+1+A2−1tanh20Ω−0.3+2A1−A2−1exp−150Ω−0.252

A1=1+ξd−1.2exp−2ξd

A2=0.5+ξd−1.0exp−2ξd

ξd=ω2dg, Ω=ωUg

In addition, in recent years, the component with the effects of forward speed is computable with an exact potential calculation.

1-3)  Eddy Making Component by Naked Hull

Ref 4)	Yoshiho Ikeda, Yoji Himeno and Norio Tanaka: On Eddy Making Component of Roll Damping Force on Naked Hull, Journal of the Society of Naval Archtects of Japan, No.142, Nov. 1977, pp.59-69.

Ref 3)	Yoshiho Ikeda, Yoji Himeno and Norio Tanaka: Component of Roll Damping of Ship at Forward Speed, Journal of the Society of Naval Architects of Japan, No.143, May 1978, pp.121-133 (in Japanese).

Ref 5)	Yoshiho Ikeda: Roll Damping, Txst of 1st symposium on stability of ships, the Society of Naval Architecs of Japan, Dec. 1984, pp.241-250.

Ref 6)	Y. Ikeda, T. Fujiwara, T. Katayama: Roll Damping of a Sharp-Cornered Barge and Roll Control by a New-Type Stabilizer, Proc. of 3rd ISOPE Vol.III, June 1993, pp.643-639.

Ref	Toru Katayama, Naofumi Yoshida, Yusuke Yamamoto, Masashi Kashiwagi, Yoshiho Ikeda: Effects of Height of Center of Rolling and Shallow Draught on Eddy Making Component of Roll Damping - Correction and Modification of Eddy Making Component in Ikeda’s method -, Proceedings of JASNAOE, No.33, Nov.2021, pp.

It is the damping component caused by vortex shedding as well as the bilge-keel component.  The local two-dimensional vortex shedding from bilge and bottom around bow and stern, where pressure gradient changes rapidly, is the main cause.  However, in this case, since KC number (Keulegan-Carpenter number) is very small, vortex shedding is not large.  Here, the pressure distribution caused on the hull surface by the vortex shedding is approximated by the first-order equation, and that pressure value is determined from the measured results of the two-dimensional models which have various shapes of cross-section.  The estimate equation for a two-dimensional section is expressed with the following equation.

f1=0.51+tanh20σ−0.7

f2=0.51−cosπσ−1.51−exp−51−σsin2πσ

Cp=0.50.87exp−γ−4exp−0.187γ+3

γ in the previous equation is the ratio of the maximum current velocity and mean velocity on the hull surface.  This can be obtained by the calculation for the Lewes form section.  Since the roll amplitude is contained in the equation of BE, the component is nonlinear component for roll amplitude.  The eddy making component decreases quickly with the increment of forward speed.  The forward speed effect is approximately expressed with the following equation.

BEB=0.50.04K0.04K2+1  where, K=LωU

1-4)  Lift Component by Naked Hull

Ref 3)	Yoshiho Ikeda, Yoji Himeno and Norio Tanaka: Component of Roll Damping of Ship at Forward Speed, Journal of the Society of Naval Architects of Japan, No.143, May 1978, pp.121-133 (in Japanese).

Under quasi-static assumption, the lift component is expressed with the following equation.

BL=0.5 ρ SL U kn l0 lR 1−1.4OGlR+0.7OGl0 lR	where l0=0.3d, lR=0.5d

The lift gradient kn to calculate maneuverability is used and it is expressed the following equation.

kn=2πdL+κ4.1BL−0.045

It should be noted that the above mention indicate only the lift component by the naked hull.  If the ship has large bilge keels or skeg and so on, these appendages may cause same component.  In this case, these components can be considered with same manner (Ref 9).

1-5)  Bilge Keel Component

Ref 7)	Yoshiho Ikeda, Yoji Himeno and Norio Tanaka: On Roll Damping Force of Ship – Effects of Friction of Hull and Normal Force of Bilge Keels -, Journal of Kansai Society of Naval Architecture, Japan, Vol.161, June  1976, pp.41-49.

Ref 8)	Yoshiho Ikeda, Kiyoshi Komatsu, Yoji Himeno and Norio Tanaka: On Roll Damping Forceof Ship – Effects of Hull Surface Pressure Created by Bilge-keels -, Journal of Kansai Society of Naval Architecture, Japan, Vol.165, June 1977, pp.31-40.

Ref 9)	Yoshiho Ikeda, Toru Katayama, Yuji Hasegawa and Masayuki Segawa: Roll Damping of High Speed Slender Vessels, Journal of Kansai Society of Naval Archtects, Japan, Vol.222, Sept. 1994, pp.73-81.

The bilge keel component is divided into normal pressure component on bilge keel and the hull surface pressure component.  The first is based on the pressure acting on the bilge keel itself, and the second is based on the pressure fluctuation caused by the vortex shedding from the bilge keel on the hull surface.  The normal pressure component on the bilge keel is given by the following equation.

BN=43πρ r3 lBK bBK φ0 ω f2 22.5bBKrπ f φ0+2.4

Where f is the modification coefficient about a bilge radius and it is expressed by the following equation.

f=1+0.3exp−1601−σ

Where, r is the distance from the center of rolling to the bilge-keel, bBK and lBK are the width and length of a bilge keel, 0 is roll amplitude.  The hull surface pressure component is calculated by integrating with the pressure distribution which a bilge keel makes along the hull surface.  In addition, two kinds of algorithms can be used in this program.  (1) is the simplified estimation method.  (2) is the method that the attachment condition of a bilge keel can be more correctly taken into consideration.  (2) should be used for the fine-ship type.

2.  INPUT DATA & OUTPUT DATA

The input file name of the following program is "OSM.RDP", an output file name is "OSMB44.CSV", and both files are text files.  The input file is written the calculated condition those are hull data, forward speed, roll amplitude, wave period and wave direction (roll period).  On the other hand, for the output file, the conditions of calculation, the calculated roll damping coefficient and its components are written.  In order to know the detail of the input file, please refer to the "SA_RDPM_new_inputfile_format.xls".

3.  ROLL CALCULATION

Ref 10)	Fukuda, J., Nagamoto, R., Konuma, M., and Takahashi, M.: "Theoretical Calculations on the Motions, Hull Surface Pressures and Transverse Strength of a Ship in Waves," Journal of the Society of Naval Architects of Japan, Japan, Vol. 129 (1971) (in Japanese).

Ref 11)	Yamanouchi, Y.: An Application of Multi-Spectrum Analysis to Ship Responses and Treatment of Nonlinear Response," Journal of the Society of Naval Architects of Japan, Vol. 125 (1969).

Ref 12)	Kaplan, P.: "Nonlinear Theory of Ship Roll Motion in a Random Seaway,"Webb !nst. of Naval Arch., Lec. Notes (1966).

Since the viscous component of a roll damping changes according to the change in roll amplitudes, total roll damping is nonlinear.

B=B1φ+B2φφ+B3φ3⋯

However, since this nonlinearity appears only for roll amplitude strongly, it can use the technique of equivalent linearization for energy in a roll motion period.

In the case of regular sea, the linearization roll damping B44 (for quadratic term) is obtained as following

B44=B1+83πωφ0B2.

The roll damping coefficient B44 is a function of roll amplitude 0, and it becomes difficult to solve roll motion equation.  The iteration method for roll amplitude is used as a method for a frequency domain roll calculation. (Ref.10)

In the case of irregular seas, to predict the ship response in short-term irregular seas, the energy-spectrum analysis based on the principle of linear superposition has been applied.  Let the energy spectrum of the incoming long-crested waves be denoted by Sw(ω).  Then we can obtain the energy spectrum of the ship response S(ω) in the form

S(ω)=A(ω)2×Sw(ω)

The frequency ω of the waves can be transformed into the encounter frequency ωe of the ship.  The response amplitude operator A(ω) represents the ship response per unit magnitude of input wave.  In particular, it becomes 0 > kha (k : the wave number) in the case of roll response.  The equation system is nonlinear when A(ω) depends on the amplitude of the wave and thus on the spectrum Sw(ω).  Several attempts have been made to treat this nonlinearity.  One of method is to take the A(ω) value is the regular perturbation technique to the nonlinear terms in the equation of motion.  The concept of the equivalent linear damping has been applied to this case:

B=B1+8πB2σφ

This has been derived from the assumption that the difference between linear and nonlinear damping can, in the sense of least-squares method, be minimized during the roll motion, which is assumed to be a Gaussian process.

δ=B1φ+B2φφ−B44φ

⇒δ2=(B1−B44)2φ2+2B2B1φ−B44φ3+B22φ4

under assumption ∂δ2∂B44=0, then B=B1φ2+B2φ3/φ2=B1+8πB2σφ

This treatment also requires an iteration method of solution, in which the variance σφof the roll angular velocity is obtained in the form

σφ2=2S(ω)ω2dω

and should be reevaluated in each iteration stage.(Ref.11)  As a more simplified way to avoid iteration calculation, it is proposed that the variance be replaced by the quantity

σφ=ωnσφ

Here, ωn is the roll natural frequency.(Ref.12)  These ways can be also applied to predict transitional time domain roll motion using convolution integration method.  However, it is should be noted that these models are proposed from the statistical point of view and are not necessarily consistent with the physical point of view.  Near future, it is hoped that a new method is proposed.

4.  COPYRIGHT, DISCLAIMER, REDISTRIBUTION OF PROGRAM

This software is freeware.  It can be used free of charge.  The program author's Toru Katayama and Yoshiho Ikeda hold copyright.  This software is not guaranteed.  No matter what trouble may occur by using this program, the authors cannot take responsibility.  Redistribution by the non-profit of this program can be freely carried out, as long as no change is carried out.  When you wish the inclusion to books, a magazine, etc., and redistribution for the profit purpose, please contact authors.  When you wish the inclusion of calculated results to journal, proceedings, etc., for research, please include "CAD system for ships, Yoshiho Ikeda and Toru Katayama, Ship and Port Editing Room (2001)", as a citation.

Yoshiho IKEDA  ikeda@marine.osakafu-u.ac.jp

Toru KATAYAMA  katayama@marine.osakafu-u.ac.jp

4.  PROGRAM LIST AND DESCRIPTION OF NOTATIONS

C**********************************************************************

C*  Estimation of Roll Damping for conventional cargo ship & barge              *

C*                                                                    *

C*  SA_RDPM.BAS                                                    *

C*                                      Osaka Prefecture University       *

C**********************************************************************

C

PROGRAM MAIN

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

DOUBLE PRECISION NUE,L,KAI,L0,L1,L2,NABLA,KW,KAI1,LTHETA,MGA,KN

&    ,LO,LR,KG,LB1,LB2,LB3,LB2S0,M1,M2,M3,M4,M5,M6,M7,M8

INTEGER BWOSM,TYPE_BK,RDP,CSDMIN,SSBK1,SSBK2

C     Max. number of square stations: SC, Max. number of points in a square station: PT

C     Max. number of wave lengths: NNRA, Max. number of Fn: NNFN

C     Max. number of wave directions: NNKAI, Max. number of roll amplitudes: NNRD

PARAMETER SC=200, PT=200, NNRA=70, NNFN=20, NNKAI=35, NNRD=30

C

DIMENSION  CSD(SC), NUMPT(SC), HT(0:SC,0:PT), HB(0:SC,0:PT)

DIMENSION  THETA1(NNRD), RAM(NNRA), FUN(NNFN), KAI(NNKAI)

DIMENSION  XX(SC,PT*2), YY(SC,PT*2)

DIMENSION  X(SC), HO(SC), SIG(SC), BX(SC), DX(SC)

DIMENSION  PERI(NNRA,NNFN,NNKAI), BE2(SC)

DIMENSION  RMAX1(2), V(2), CR(SC), CR1(SC)

DIMENSION  X1(30), YI(30), XBK(0:SC), RATIO(SC)

DIMENSION  X11(SC), Y11(SC), WX(SC), WY(SC)

DIMENSION  BKT(SC), BKB(SC), NUBK(SC), BBKHAT(SC)

DIMENSION  BBYD(SC), SGM(SC), BB0(SC), TT0(SC), WLHB(SC), WLAR(SC)

DIMENSION  GLEN(SC)

C

C

C

C******

C    if BWOSM=0, wave making comp is not calculated.

C    if BWOSM=1, wave making comp is calculated by simplified calculation.

BWOSM=0

C     Coefficient of kinematic viscosity: NUE

NUE=1.14E-06

C     Circular constant : PAI

PAI=3.14159

C     Gravitational acceleration: G, Fluid density: RO

G=9.8

RO=102.0

C

C     loading the data file which is written hull and bilge keel data and sea conditions: OSM.RDP

OPEN(1, FILE='OSM.RDP')

C****** Hull form data

C     Lpp[m]：L、Breadth[m]：B、Draft[m]：D

READ(1,*)L, B, D

C     number of cross section in offset data: MUMCS, the number indicating mid-ship section: IMID

READ(1,*)NUMCS, IMID

DO I=1, NUMCS

C     SS number：CSD(I)

READ(1,*) CSD(I)

C     number of datum in a square section: NUMPT(I)

READ(1,*) NUMPT(I)

DO J=1 , NUMPT(I)

C     height from Base Line[m]: HT(I,J), breadth from Center Line[m]: HB(I,J) (please input from BL & CL）

READ(1,*) HT(I,J), HB(I,J)

END DO

END DO

C****** information of bilge keel

C     breadth of bilge keel [m]: BBK, bilge keel aft-end ss number: XBK1, bilge keel forward-end ss number: XBK2

READ(1,*) BBK, XBK1, XBK2

C     Selection of calculation methods for bilge keel component (TYPE_BK= 1: simplified, 2: integration alone hull form)

READ(1,*) TYPE_BK

C     Attention) if TYPE_BK=2 them, the datum from “IF ( TYPE_BK.NE.1) then” to “END IF” are loaded.

IF (TYPE_BK.NE.1) then

C     The aft-end and the forward-end of the section data number (1 to MUMCS) in the program where the bilge keel exists on hull.: SSBK1 & SSBK2 ( 1 to MUMCS)

READ(1,*) SSBK1, SSBK2

DO I=SSBK1, SSBK2

C     The attached transverse position of the bilge-keel on the section from CL [m]：BKB(I)

READ(1,*) BKB(I)

END DO

END IF

C*****Selection of calculation methods for eddy making component (RDP=1: conventional hull, 2:barge)

READ(1,*) RDP

C*****OGD=(Draft-KG)/Draft

READ(1,*) OGD

C*****Number of roll amplitudes for calculation: NTHETA

READ(1,*) NTHETA

DO I=1 , NTHETA

C*****Roll amplitude [degree]：THETA1(I)

READ(1,*) THETA1(I)

END DO

C*****Number of Froude numbers (Fn) forculculation: NFN

READ(1,*) NFN

DO I=1 , NFN

C*****Froude number (Fn)：FUN(I)

READ(1,*) FUN(I)

END DO

C*****Number of wavelength/Lpp for calculation: NRA

READ(1,*) NRA

DO I=1, NRA

C*****wavelength/Lpp: RAM(I)

READ(1,*) RAM(I)

END DO

C*****Number of wave directions for calculation: NKAI

READ(1,*) NKAI

DO I=1, NKAI

C*****wave direction(degree),  0=following sea, 180=head sea: KAI(I)

READ(1,*) KAI(I)

END DO

CLOSE(1)

C=====

C

C=====When a bilge-keel component is calculated by the integration method, OFFSET DATA is extended to right and left.

DO I=1 , NUMCS

HT(I,NUMPT(I)+1)=999

HB(I,NUMPT(I)+1)=HB(I,NUMPT(I))

END DO

DO I=1 , NUMCS

DO J=1 , NUMPT(I)

XX(I,NUMPT(I)-J+1)=HB(I,J)

XX(I,NUMPT(I)+J-1)=-HB(I,J)

YY(I,NUMPT(I)-J+1)=HT(I,J)

YY(I,NUMPT(I)+J-1)= HT(I,J)

END DO

END DO

C

C====== calculation of hull form data =====

C=====half breadth：WLHB(I)、water line area：WLAR(I)、girth length：GLEN(I)

WLHT=D

DO I=1 , NUMCS

SUM=0.0

10    CONTINUE

IF( HT(I,1).GT.WLHT)THEN

WLHB(I)=0.

WLAR(I)=0.

GLEN(I)=0.

GOTO 10

END IF

IF( HT(I,1).EQ.WLHT)THEN

WLHB(I)=HB(I,1)

WLAR(I)=0.

GLEN(I)=HB(I,1)

END IF

DO J=1 , NUMPT(I)

11    CONTINUE

IF( (HT(I,J).LT.WLHT) .AND. (WLHT.LE.HT(I,J+1)))THEN

GOTO 12

END IF

SUM=SUM+(HT(I,J+1)-HT(I,J))*(HB(I,J+1)+HB(I,J))/2

GOTO 14

C      ====== calculation on each water line

12    CONTINUE

AA=(HB(I,J+1)-HB(I,J))/(HT(I,J+1)-HT(I,J))

BB=(-HB(I,J+1)*HT(I,J)+HB(I,J)*HT(I,J+1))/(HT(I,J+1)-HT(I,J))

WLHB(I)=AA*WLHT+BB

WLAR(I)=SUM+(WLHT-HT(I,J))*(WLHB(I)+HB(I,J))/2

GOTO 15

14    CONTINUE

END DO

15    CONTINUE

END DO

C      =====

C     H0: BBYD(I), σ: SGM(I), draft of a section/mean draft: TT0(I)

C     water line breadth of a section/Breadth: BB0(I)

DO I=1, NUMCS

IF (HT(1,1).LT.D) THEN

CSDMIN=1

END IF

IF ((HT(I-1,1).GT.D).AND.(HT(I,1).GE.D)) THEN

GOTO21

END IF

IF ((HT(I-1,1).GE.D).AND.(HT(I,1).LT.D)) THEN

CSDMIN=I

END IF

IF ((HT(I,1).GE.D).AND.(HT(I-1,1).LT.D)) THEN

NUMH0=I-1

GOTO 22

END IF

HB2=HB(I,1)

DO J=2, NUMPT(I)

IF (HT(I,J).GT.WLHT) THEN

GOTO 18

END IF

IF (HB(I,J).GT.HB2) THEN

HB2=HB(I,J)

END IF

END DO

18      CONTINUE

IF (WLHB(I).GT.HB2) THEN

HB2=WLHB(I)

END IF

IF (WLHB(I).EQ.0) THEN

BBYD(I)=HB2/(WLHT-HT(I,1))

GOTO 19

END IF

BBYD(I)=WLHB(I)/(WLHT-HT(I,1))

19      CONTINUE

IF (WLHB(I).EQ.0) THEN

SGM(I)=WLAR(I)/(HB2*(WLHT-HT(I,1)))

GOTO 20

END IF

SGM(I)=WLAR(I)/(WLHB(I)*(WLHT-HT(I,1)))

20      CONTINUE

TT0(I)=(WLHT-HT(I,1))/D

IF (TT0(I).LT.0) THEN

TT0(I)=0

END IF

IF (WLHB(I).EQ.0) THEN

BB0(I)=HB2/B*2

GOTO 21

END IF

BB0(I)=WLHB(I)/B*2

21      CONTINUE

END DO

NUMH0=NUMCS

22    CONTINUE

C

N=NUMCS-CSDMIN+1

DO I=1, N

X(I)  =CSD(CSDMIN-1+I)

HO(I) =BBYD(CSDMIN-1+I)

SIG(I)=SGM(CSDMIN-1+I)

BX(I) =BB0(CSDMIN-1+I)*B

DX(I) =TT0(CSDMIN-1+I)*D

END DO

C

C====== calculation of displacemental volume

C      displacemental volume [m3]：NABLA

ODD=0.0

if (mod(NUMCS,2).LT.0.5) then

ODD=1.0

end if

DO J=1, NUMCS-2-ODD , 2

Q0=CSD(J)

Q1=CSD(J+1)

Q2=CSD(J+2)

QQ=(Q0+Q2)/2

HH=(Q2-Q0)/2*L/10/3

L0=4*(QQ-Q1)*(QQ-Q2)/(Q0-Q1)/(Q0-Q2)+1

L1=4*(QQ-Q0)*(QQ-Q2)/(Q1-Q0)/(Q1-Q2)

L2=4*(QQ-Q0)*(QQ-Q1)/(Q2-Q0)/(Q2-Q1)+1

SUM3=SUM3+(L0*WLAR(J)+L1*WLAR(J+1)+L2*WLAR(J+2))*HH

END DO

IF (ODD.LT.0.5) THEN

GOTO 1404

END IF

J=NUMCS-1

HH=(CSD(J+1)-CSD(J))/2*L/10

SUM3=SUM3+(WLAR(J)+WLAR(J+1))*HH

1404  NABLA=2*SUM3

C

C====== calculation of block coefficient Cb: CB, midship coefficient CM: CM

CB=NABLA/L/B/D

CM=2*WLAR(IMID)/B/D

C

C====== Calculation of the encounter period for calculating roll damping

C      wave length λ [m]: RAM(I), Fn: FUN(J), wave direction χ [deg]: KAI(K)

C      encounter frequency [rad/sec]: MGA, encounter period [sec]: PERI(I,J,K)

DO K=1, NKAI

DO J=1, NFN

DO I=1, NRA

OMG=SQRT(2*PAI*G/RAM(I)/L)

KW=2*PAI/RAM(I)/L

V2=FUN(J)*SQRT(L*G)

KAI1=KAI(K)*PAI/180

MGA=OMG-KW*V2*COS(KAI1)

MGA=ABS(MGA)

PERI(I,J,K)=2*PAI/MGA

END DO

END DO

END DO

C

C=====The attachment height of a bilge keel is calculated using a bilge-keel transverse direction attachment position and hull form data. [m]: BKT(I)

C=====In the data in a section, it is verified whether the bilge keel is attached between the data of what position.

IF (TYPE_BK.NE.1)THEN

DO I=SSBK1, SSBK2

J=1

3         CONTINUE

NUBK(I)=NUMPT(I)-J+1

IF (HB(I,J).LE.BKB(I)) THEN

J=J+1

GOTO 3

END IF

BKT(I)=(HT(I,J)-HT(I,J-1))*(BKB(I)-HB(I,J-1))

&            /(HB(I,J)-HB(I,J-1))+HT(I,J-1)

END DO

END IF

C

C*********************************************************************

C       Output file of the calculated results: OSMB44.CSV

OPEN(2,FILE='OSMB44.CSV')

NO=0

DO ITHETA=1, NTHETA

THTA=THETA1(ITHETA)*3.14159/180

DO IKAI=1, NKAI

DO IFN=1, NFN

DO IRAM=1, NRA

NO=NO+1

C

OMEGA=6.28318/PERI(IRAM,IFN,IKAI)

C

C      *********** Frictional Component ( frictional component) ************

C      Ref. H.Kato (JZK. No.102) and S.Tamiya et al (JZK. No.132)

C

SF=L*(1.7*D+CB*B)

RF=((0.887+0.145*CB)*(1.7*D+CB*B)-2*OGD*D)/3.1415

BFHAT=0.787*SF*RF**2.*SQRT(OMEGA*NUE*B/19.6133)

&         /(NABLA*B**2.)*(1.+4.1*FUN(IFN)/OMEGA*SQRT(9.80665/L))

C

C

C      ********** Wave Making Component (wave making component) **********

C      Generally, this component should be calculated by the external potential code.

C      Ref. Y.Ikeda et al (JZK. No.143)

C

GUZAID=OMEGA**2.*D/9.80665

A1=1+GUZAID**(-1.2)*EXP(-2*GUZAID)

A2=0.5+GUZAID**(-1)*EXP(-2*GUZAID)

LOMEGA=OMEGA*FUN(IFN)*SQRT(L/9.80665)

SIT=20.*(LOMEGA-0.3)

TAH=(EXP(SIT)-EXP(-SIT))/(EXP(SIT)+EXP(-SIT))

BWHAT=BWOSM*.5*(((A2+1)+(A2-1)*TAH)

&        +(2*A1-A2-1)*EXP(-150*(LOMEGA-.25)**2))

C

C

C     ********** Lift Component **********

C      Ref. Y.Ikeda et al (JZK. No.143)

C

IF( CM.LE.0.92 )THEN

KAPA=0.0

END IF

IF( (CM.LE.0.97).AND.(CM.GT.0.92) )THEN

KAPA=0.1

END IF

IF( CM.GT.0.97 )THEN

KAPA=0.3

END IF

KN=6.28319*D/L+KAPA*(4.1*B/L-0.045)

OG=OGD*D

LO=0.3*D

LR=0.5*D

BLHAT=L*D*KN*LO*LR*FUN(IFN)*0.5/(NABLA*B**2)

&        *SQRT(0.5*L*B)*(1-1.4*OG/LR+0.7*OG**2/(LO*LR))

C

C      ******* Eddy Making Component ********

C             RDP=3: for barge, RDP=1: for conventional round hull

IF (RDP.EQ.3) THEN

C

C      ******* Eddy Making Component for type barge ships *******

C       Ref. Y.Ikeda  et al(Proc. Ship Motion, Wave Loads and

C                          Propaulsive Performance in a Seaway,1984)

C          Y.Ikeda, T.Fujiwara, T.Katayama (Proc ISOPE Vol.3, 1993)

C

OG=OGD*D

DO I=1, N

BE2(I)=(2/3.141592)*DX(I)**4/NABLA/BX(I)**2*(B/2/9.8)**.5

&           *(HO(I)**2+1-OG/DX(I))*(HO(I)**2

&           +(1-OG/DX(I))**2)*THTA*OMEGA

END DO

BEHAT=0

DO I=1,N-1

XHA=(X(I+1)-X(I))

SBE=XHA*(BE2(I)+BE2(I+1))/2

BEHAT=BEHAT+SBE

END DO

BEHAT=BEHAT*L/(X(N)-X(1))

IF (FUN(IFN).EQ.0) then

BEHAT=BEHAT

GOTO 5215

END IF

AK=OMEGA/FUN(IFN)*SQRT(L/9.8)

BEHAT=BEHAT*(.04*AK)**2/((.04*AK)**2+1)

5215   CONTINUE

C

C

GOTO 5064

END IF

C

C     *********** Eddy Making Component **********

C     Ref. Y.Ikeda et al (JZK. No.142, JZK. No.143)

C     Ref.   T.Katayama et al  (PJASNAOE No.33)

C

DO J=1, N

AHO=HO(J)/(1-OGD)

SGUMA=(SIG(J)-OGD)/(1-OGD)

E=(AHO-1)/(AHO+1)

E2=E**2

A=4*SGUMA*(1-E2)/3.1415+E2

O=-A/(A+3)

O2=SQRT(O**2-(A-1)/(A+3))

A3=O+O2

A1=E*(1+A3)

AM=BX(J)/(1+A1+A3)*0.5

AA1=A1*(1+A3)/A3*0.25

IF (AA1.GT.1) THEN

AA1=1

END IF

IF (AA1.LE.-1) THEN

AA1=-1

END IF

DO I=1, 2

CALL ARC(AA1,ACS)

LTHETA=.5*ACS

IF (I.EQ.1) THEN

LTHETA=0

END IF

AH=1+A1**2+9*A3**2+2*A1*(1-3*A3)*COS(2*LTHETA)

&         -6*A3*COS(4*LTHETA)

AA=-2*A3*COS(5*LTHETA)+A1*(1-A3)*COS(3*LTHETA)

&         +((6-3*A1)*A3**2+(A1**2-3*A1)*A3+A1**2)*COS(LTHETA)

BB=-2*A3*SIN(5*LTHETA)+A1*(1-A3)*SIN(3*LTHETA)

&         +((6+3*A1)*A3**2+(3*A1+A1**2)*A3+A1**2)*SIN(LTHETA)

V(I)=2*AM*SQRT(AA**2+BB**2)/AH

RMAX1(I)=AM*SQRT(((1+A1)*SIN(LTHETA)-A3*SIN(3*LTHETA))**2

&               +((1-A1)*COS(LTHETA)+A3*COS(3*LTHETA))**2)

END DO

RMAX=RMAX1(1)

VMAX=V(1)

IF (RMAX1(1).LE.RMAX1(2)) THEN

GOTO 5159

END IF

GOTO 5161

5159     RMAX=RMAX1(2)

VMAX=V(2)

5161     RMEAN=2*DX(J)*(1-OGD)*SQRT(AHO*SGUMA/3.1415)

P1=VMAX/RMEAN

P2=RMAX/RMEAN

PP3=P1+P2

GAMMA=(1+4*EXP(-165000*(1-SGUMA)**2))*PP3

CP=0.5*(0.87*EXP(-GAMMA)-4*EXP(-0.187*GAMMA)+3)

SIT=20*(SIG(J)-0.7)

CALL TANH(TAH,SIT)

F1=0.5*(1+TAH)

F2=0.5*(1-COS(3.1415*SIG(J)))-1.5*(1-EXP(-5*(1-SIG(J))))

&       *SIN(3.1415*SIG(J))**2

IF (SIG(J).GT.1) THEN

SIG(J)=1.

END IF

R=2*DX(J)*SQRT(HO(J)*(SIG(J)-1)/(-0.8584))

RD=R/DX(J)

IF ((HO(J).LE.1).AND.(RD.GE.AHO)) THEN

R=.5*BX(J)

END IF

IF ((HO(J).GE.1).AND.(RD.GE.1)) THEN

R=DX(J)

END IF

RD=R/DX(J)

CR1(J)=RMAX**2/DX(J)**2*CP*((1-F1*RD)*(1-3/2*OGD-F1*RD)

&          +F2*(HO(J)-F1*RD)**2)

IF( CR1(J).LT.0 )THEN

CR1(J)=0

END IF

END DO

DO K=1, 21

X1(K)=0+.5*(K-1)

MAX=N

MAXX=SC

NN=MAX

M1=1

M2=0

XXX=X1(K)

DO JJ=1, MAXX

X11(JJ)=X(JJ)

Y11(JJ)=CR1(JJ)

END DO

CALL HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

CR(K)=Y2

DAM=YX

END DO

CR(1)=1.5*(1-3/2*OGD)

CR(21)=1.5*(1-3/2*OGD)

SAM=0

DO K=1, 10

K2=2*K

K1=K2-1

K3=K2+1

SAM1=CR(K1)+4*CR(K2)+CR(K3)

SAM=SAM+SAM1

END DO

CRT=SAM/60

BEHAT=4*L*D**4/3/3.1415*OMEGA*SQRT(B/19.6)/NABLA/B**2*CRT*THTA

DO ISSN=1,N

BE2(ISSN)=CR1(ISSN)*4*D**4/3/3.1415*OMEGA

&              *SQRT(B/19.6)/NABLA/B**2*THTA

END DO

C

IF (FUN(IFN).EQ.0) then

BEHAT=BEHAT

GOTO 5211

END IF

C

AK=OMEGA/FUN(IFN)*SQRT(L/9.8)

BEHAT=BEHAT*(.04*AK)**2/((.04*AK)**2+1)

DO ISSN=1,N

BE2(ISSN)=BE2(ISSN)*(.04*AK)**2/((.04*AK)**2+1)

END DO

5211   CONTINUE

C

C

C =====Start of calculation of Bilge keel component======

C

5064   IF (BBK.LE.1E-10) THEN

GOTO 5067

END IF

C

C      TYPE_BK=1: simplified method

C      TYPE_BK=2: integration method

C

IF (TYPE_BK.EQ.1) THEN

C

C      ********** Damping due to Bilge Keels  (The simplified estimation method)**********

C      Ref. Y.Ikeda et al (JZK. No.161 , JZK. No.165)

C

XBK(1)=XBK1

XBK(11)=XBK2

DO I=2, 10

XBK(I)=XBK(I-1)+(XBK2-XBK1)*.1

END DO

MAX=N

DO I=1, 11

MAXX=SC

NN=MAX

M1=1

M2=0

XXX=XBK(I)

DO JJ=1, MAXX

X11(JJ)=X(JJ)

Y11(JJ)=HO(JJ)

END DO

CALL HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

HO1=Y2

DAM=YX

MAXX=SC

NN=MAX

M1=1

M2=0

XXX=XBK(I)

DO JJ=1, MAXX

X11(JJ)=X(JJ)

Y11(JJ)=SIG(JJ)

END DO

CALL HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

SG1=Y2

DAM=YX

MAXX=SC

NN=MAX

M1=1

M2=0

XXX=XBK(I)

DO JJ=1, MAXX

X11(JJ)=X(JJ)

Y11(JJ)=DX(JJ)

END DO

CALL HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

DX1=Y2

DAM=YX

MAXX=SC

NN=MAX

XXX=XBK(I)

M1=1

M2=0

XXX=XBK(I)

DO JJ=1, MAXX

X11(JJ)=X(JJ)

Y11(JJ)=BX(JJ)

END DO

CALL HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

BX1=Y2

DAM=YX

R=2*DX1*sqrt(HO1*(SG1-1)/(-.8585))

RD=R/DX1

IF ((HO1.LE.1).AND.(RD.GE.HO1)) then

R=.5*BX1

END IF

IF ((HO1.GT.1).AND.(RD.GE.1)) then

R=DX1

END IF

RD=R/DX1

F=1+.3*EXP(-160*(1-SG1))

RBK=DX1*SQRT((HO1-.2929*RD)**2+(1-OGD-.2929*RD)**2)

M1=RD

M2=OGD

M3=1-M1-M2

M4=HO1-M1

M5=(.414*HO1+.0651*M1**2-(.382*HO1+.0106)*M1)

&       /((HO1-.215*M1)*(1-.215*M1))

M6=(.414*HO1+.0651*M1**2-(.382+.0106*HO1)*M1)

&       /((HO1-.215*M1)*(1-.215*M1))

SO=.3*(3.1415*F*RBK*THTA)+1.95*BBK

M7=SO/DX1-.25*3.1415*M1

R1=.25*3.1415*R

IF (SO.LT.R1) then

M7=0

END IF

M8=M7+.414*M1

IF (SO.LT.R1) then

M8=M7+1.414*(1-cos(SO/R))*M1

END IF

A=(M3+M4)*M8-M7**2

BB=M4**3/3/(HO1-.215*M1)+(1-M1)**2*(2*M3-M2)

&       /6/(1-.215*M1)+M1*(M3*M5+M4*M6)

CPPLAS=1.2

CPMINS=-22.5*BBK/(3.1415*RBK*F*THTA)-1.2

CD=CPPLAS-CPMINS

C

C      *** BBKhat for unit length ***

C

RATIO(I)=RBK*BBK*CD/(RBK*BBK*CD+.5*DX1**2

&           *(-A*CPMINS+BB*CPPLAS))

BBKHAT(I)=8*RBK**2*OMEGA*SQRT(B/19.6)

&              *THTA*F**2/(3*3.1415*NABLA*B**2)

&              *(RBK*BBK*CD+.5*DX1**2*(-A*CPMINS+BB*CPPLAS))

END DO

C

C      *** BBKhat for Three Dimentional Ship Form ***

C

SAM=0

DO I=1,5

I2=2*I

SAM1=BBKHAT(I2-1)+4*BBKHAT(I2)+BBKHAT(I2+1)

SAM=SAM+SAM1

END DO

BKHAT=SAM*(XBK2-XBK1)*.1/3*L*.1

C

C

ELSE

C

C      ********** Damping due to Bilge Keels **********

C      Pressure integration along a hull surface

C      Ref. Y.Ikeda, T.Katayama et al (JKSNAJ. No.222)

C

BBK0=0

BBK1=0

BBK2=0

BBK3=0

RO=102

KG=(1-OGD)*D

DO P=SSBK1, SSBK2

SGUMA=(SIG(P)-OGD)/(1-OGD)

C      ******* normal pressure component *******

C      ******* F1, R, CD----->FBN

K=NUBK(P)

XX0=XX(P,K)

XX1=BKB(P)

XX2=XX(P,K+1)

YY0=YY(P,K)

YY1=BKT(P)

YY2=YY(P,K+1)

IF (XX2.EQ.XX1) THEN

XX2=XX(P,K+2)

LOW=102

END IF

YY2=YY(P,K+2)

F1=1+.3*EXP((-1)*160*(1-SGUMA))

R=SQRT((KG-YY1)**2+XX1**2)

CD=22.5*BBK/(3.14159*F1*R*THTA)+2.4

FBN=8/3*RO*R**2*BBK*THTA*F1**2*CD*OMEGA/6.28

C       ******* XA,YA ***** Gradient of hull surface.  Quadratic function approximation

IF (XX0.EQ.XX2) THEN

XA=0

YA=YY1

GOTO 3550

END IF

IF (YY0.EQ.YY2) THEN

XA=XX1

YA=KG

GOTO 3550

END IF

FD0=YY0/((XX0-XX1)*(XX0-XX2))

FD1=YY1/((XX1-XX0)*(XX1-XX2))

FD2=YY2/((XX2-XX0)*(XX2-XX1))

FD=2*XX1*(FD0+FD1+FD2)-(XX1+XX2)

&          *FD0-(XX0+XX2)*FD1-(XX0+XX1)*FD2

XA=(XX1+FD*(YY1-KG))/(1+FD**2)

YA=(FD*XX1+FD**2*(YY1-KG)+KG)/(1+FD**2)

C        ******* LB1,BN

3550        LB1=SQRT((XA-XX1)**2+(YA-YY1)**2)

BN=FBN*LB1*2

C

C        ******* negative pressure component *******

C        ******* S0,CPN----->FBSN

S0=.3*3.14159*F1*R*THTA+1.95*BBK

CPN=1.2-CD

FBSN=8/3*RO*R**2*F1**2*THTA*CPN*OMEGA/6.28

C        ******* starboard side

C        ******* XX0,YY0,XX1,YY1

MPN=0

LB2S0=0

OWARI=0

BSN=0

BSNS=0

BSNP=0

K=NUBK(P)

XX0=BKB(P)

YY0=BKT(P)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

GOTO 3950

3900        K=K+1

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

GOTO 3950

3920        XX1=(XX1-XX0)*(S0+LB2-LB2S0)/LB2+XX0

YY1=(YY1-YY0)*(S0+LB2-LB2S0)/LB2+YY0

C         ******* XM,YM,XA,YA

3950        XM=(XX0+XX1)/2

YM=(YY0+YY1)/2

IF (XX0.EQ.XX1) THEN

XA=0

YA=YM

GOTO 4010

END IF

IF (YY0.EQ.YY1) THEN

XA=XM

YA=KG

GOTO 4010

END IF

XA=(YM+(XX0-XX1)/(YY0-YY1)*XM-KG)*(XX0-XX1)

&          *(YY0-YY1)/((YY0-YY1)**2+(XX0-XX1)**2)

YA=(YY0-YY1)/(XX0-XX1)*XA+KG

C         ******* LB2,LB3----->BSNS

4010        LB2=SQRT((XX0-XX1)**2+(YY0-YY1)**2)

LB2S0=LB2S0+LB2

IF ((OWARI.EQ.0).AND.(S0.LT.LB2S0)) THEN

OWARI=1

GOTO 3920

END IF

LB3=SQRT((XM-XA)**2+(YM-YA)**2)

BSNS=BSNS+FBSN*LB2/LB3*(XA*(YM-YA)-(YA-KG)*(XM-XA))

IF (OWARI.EQ.1) THEN

OWARI=0

GOTO 4110

END IF

GOTO 3900

C         ******* port side

C         ******* XX0,YY0,XX1,YY1

4110        LB2S0=0

K=2*NUMPT(P)-1-NUBK(P)

XX0=-BKB(P)

YY0=BKT(P)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

GOTO 4160

4140        K=K+1

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

4160        IF (YY1.GT.D) THEN

OWARI=1

XX1=(XX1-XX0)*(D-YY0)/(YY1-YY0)+XX0

YY1=D

END IF

GOTO 4210

4180        XX1=(XX1-XX0)*(S0+LB2-LB2S0)/LB2+XX0

YY1=(YY1-YY0)*(S0+LB2-LB2S0)/LB2+YY0

C         ******* XM,YM,XA,YA

4210         XM=(XX0+XX1)/2

YM=(YY0+YY1)/2

IF (XX0.EQ.XX1) THEN

XA=0

YA=YM

GOTO 4270

END IF

IF (YY0.EQ.YY1) THEN

XA=XM

YA=KG

GOTO 4270

END IF

XA=(YM+(XX0-XX1)/(YY0-YY1)*XM-KG)*(XX0-XX1)

&          *(YY0-YY1)/((YY0-YY1)**2+(XX0-XX1)**2)

YA=(YY0-YY1)/(XX0-XX1)*XA+KG

C         ******* LB2,LB3----->BSNP

4270        LB2=SQRT((XX0-XX1)**2+(YY0-YY1)**2)

LB2S0=LB2S0+LB2

IF ((OWARI.EQ.0).AND.(S0.LT.LB2S0)) THEN

OWARI=1

GOTO 4180

END IF

LB3=SQRT((XM-XA)**2+(YM-YA)**2)

BSNP=BSNP+FBSN*LB2/LB3*(XA*(YM-YA)-(YA-KG)*(XM-XA))

IF (OWARI.EQ.1) THEN

OWARI=0

GOTO 4350

END IF

GOTO 4140

4350      BSN=BSNS+BSNP

C

C      ******* positive pressure component *******

C      ******* CPP

CPP=1.2

C      ******* starboard side

C      ******* GLWBK

GLWBK=0

GLBKK0=0

GLBKK1=0

BSP=0

BSPS=0

BSPP=0

DO K=1, NUBK(P)

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

IF (YY1.GE.D) THEN

goto 4520

END IF

IF ((YY1.LT.D).AND.(YY0.GE.D)) then

XX0=(XX1-XX0)*(D-YY0)/(YY1-YY0)+XX0

YY0=D

END IF

C

IF (K.EQ.NUBK(P)) THEN

XX1=BKB(P)

YY1=BKT(P)

END IF

GLWBK=GLWBK+SQRT((XX0-XX1)**2+(YY0-YY1)**2)

4520      END DO

C      ******* GLBKK0, GLBKK1, FBSP0, FBSP1

DO K=1, NUBK(P)

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

IF (YY1.GE.D) then

goto 4740

END IF

IF ((YY1.LT.D).AND.(YY0.GE.D)) then

XX0=(XX1-XX0)*(D-YY0)/(YY1-YY0)+XX0

YY0=D

END IF

IF (K.EQ.NUBK(P)) THEN

XX1=BKB(P)

YY1=BKT(P)

END IF

GLBKK0=GLBKK1

GLBKK1=GLBKK0+SQRT((XX0-XX1)**2+(YY0-YY1)**2)

FBSP0=8/3*RO*R**2*THTA*F1**2*CPP*OMEGA/6.28*GLBKK0/GLWBK

FBSP1=8/3*RO*R**2*THTA*F1**2*CPP*OMEGA/6.28*GLBKK1/GLWBK

C      ******* XM, YM, XA, YA, LB2, LB3----->BSP

XM=(FBSP0+2*FBSP1)/(3*FBSP0+3*FBSP1)*(XX1-XX0)+XX0

YM=(FBSP0+2*FBSP1)/(3*FBSP0+3*FBSP1)*(YY1-YY0)+YY0

IF (XX0.EQ.XX1) THEN

XA=0

YA=YM

GOTO 4700

END IF

IF (YY0.EQ.YY1) THEN

XA=XM

YA=KG

GOTO 4700

END IF

XA=(YM+(XX0-XX1)/(YY0-YY1)*XM-KG)*(XX0-XX1)

&          *(YY0-YY1)/((YY0-YY1)**2+(XX0-XX1)**2)

YA=(YY0-YY1)/(XX0-XX1)*XA+KG

4700        LB2=SQRT((XX0-XX1)**2+(YY0-YY1)**2)

LB3=SQRT((XM-XA)**2+(YM-YA)**2)

BSPS=BSPS+(FBSP0+FBSP1)/2*LB2/LB3

&            *(XA*(YM-YA)-(YA-KG)*(XM-XA))

4740      END DO

C       ******* port side

C       ******* GLWBK

GLWBK=0

GLBKK0=0

GLBKK1=0

OWARI=0

KSTA=NUMPT(P)

KEND=2*NUMPT(P)-1-NUBK(P)

DO K=KSTA, KEND

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

IF (K.EQ.KEND) THEN

XX1=-BKB(P)

YY1=BKT(P)

END IF

GLWBK=GLWBK+SQRT((XX0-XX1)**2+(YY0-YY1)**2)

END DO

C       ******* GLBKK0, GLBKK1, FBSP0, FBSP1

DO K=KSTA, KEND

XX0=XX(P,K)

YY0=YY(P,K)

XX1=XX(P,K+1)

YY1=YY(P,K+1)

IF (K.EQ.KEND) THEN

XX1=-BKB(P)

YY1=BKT(P)

END IF

GLBKK0=GLBKK1

GLBKK1=GLBKK0+SQRT((XX0-XX1)**2+(YY0-YY1)**2)

FBSP0=8/3*RO*R**2*THTA*F1**2*CPP*OMEGA/6.28*GLBKK0/GLWBK

FBSP1=8/3*RO*R**2*THTA*F1**2*CPP*OMEGA/6.28*GLBKK1/GLWBK

C       ******* XM, YM, XA, YA, LB2, LB3----->BSP

XM=(FBSP0+2*FBSP1)/(3*FBSP0+3*FBSP1)*(XX1-XX0)+XX0

YM=(FBSP0+2*FBSP1)/(3*FBSP0+3*FBSP1)*(YY1-YY0)+YY0

IF (XX0.EQ.XX1) THEN

XA=0

YA=YM

GOTO 4990

END IF

IF (YY0.EQ.YY1) THEN

XA=XM

YA=KG

GOTO 4990

END IF

XA=(YM+(XX0-XX1)/(YY0-YY1)*XM-KG)*(XX0-XX1)

&          *(YY0-YY1)/((YY0-YY1)**2+(XX0-XX1)**2)

YA=(YY0-YY1)/(XX0-XX1)*XA+KG

4990        LB2=SQRT((XX0-XX1)**2+(YY0-YY1)**2)

LB3=SQRT((XM-XA)**2+(YM-YA)**2)

BSPP=BSPP+(FBSP0+FBSP1)/2*LB2/LB3

&             *(XA*(YM-YA)-(YA-KG)*(XM-XA))

END DO

BSP=BSPS+BSPP

C

C********************************************************************

BBK0=BBK1

BBK1=BN+BSN+BSP

BBK2=BBK0+BBK1

IF (P.EQ.SSBK1) then

BBK3=BBK3+L*(CSD(P)-XBK1)/10*BBK1

GOTO 5140

END IF

BBK3=BBK3+L*(CSD(P)-CSD(P-1))/10*BBK2/2

5140      IF (P.EQ.SSBK2) THEN

BBKDAMP=BBK3+L*(XBK2-CSD(P))/10*BBK1

END IF

C       *******

END DO

BKHAT=BBKDAMP/(RO*NABLA*B**2)*SQRT(B/19.6)

C       *******

C=======================================

END IF

B2KHAT=BKHAT

5067           IF( BBK.LE.1E-10)THEN

B2KHAT=0.

END IF

C =====Here, the program about calculation of a bilge-keel component finishes.======

C      Below, a calculated result is written out to a file.

BB44H=BFHAT+BWHAT+BLHAT+BEHAT+B2KHAT

B44   =BB44H*(RO*NABLA*B**2)/SQRT(B/19.6)

BF    =BFHAT *(RO*NABLA*B**2)/SQRT(B/19.6)

BW   =BWHAT *(RO*NABLA*B**2)/SQRT(B/19.6)

BL    =BLHAT *(RO*NABLA*B**2)/SQRT(B/19.6)

BE    =BEHAT *(RO*NABLA*B**2)/SQRT(B/19.6)

BBBK =B2KHAT*(RO*NABLA*B**2)/SQRT(B/19.6)

C=========== The write-in line to the file of a roll damping calculated result.  Below, it is shown in order of writing.

C               roll amp.[deg]、Fn、wave length: λ[m]、wave direction: χ[deg]

B44C              non-D roll damping coef.: B44、frictional comp.: BF、wave making comp.: BW

C                           lift comp.: BL、eddy making comp.: BE、bilgekeel comp.: BBK

C             dimensional damping coef.: B44、frictional comp.: BF、wave making comp.: BW

C                           lift comp.: BL、eddy making comp.: BE、bilgekeel comp.: BBK

WRITE(2,100) THETA1(ITHETA), FUN(IFN), RAM(IRAM)

&                  ,KAI(IKAI), BB44H, BFHAT, BWHAT, BLHAT, BEHAT

&                  , B2KHAT, B44, BF, BW, BL, BE, BBBK

100           FORMAT(16(F20.9,','))

C============

END DO

END DO

END DO

END DO

END DO

CLOSE(2)

STOP

END

C

C

C

C       ************   Subroutine   *************

C      ********** Lagrange 3 Points Interpolation **********

SUBROUTINE HOKAN1(NN,IL,XXX,YX,Y2,M1,M2,X11,Y11,WX,WY)

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

DOUBLE PRECISION NUE,L,KAI,L0,L1,L2,NABLA,KW,KAI1,LTHETA,MGA,KN

&    ,LO,LR,KG,LB1,LB2,LB3,LB2S0,M1,M2,M3,M4,M5,M6,M7,M8

INTEGER BWOSM,TYPE_BK,RDP,CSDMIN,SSBK1,SSBK2

PARAMETER SC=200,PT=200,NNRA=70,NNFN=20,NNKAI=35,NNRD=30

C

DIMENSION  X11(SC),Y11(SC),WX(SC),WY(SC)

C

C

N1=NN-1

DO IL=2,N1

IF(XXX.LE.X11(IL)) THEN

GOTO 5324

END IF

END DO

5324    I1=IL-1

IF (XXX.GT.X11(N1)) THEN

I1=NN-2

END IF

I2=I1+2

DO IL=I1,I2

II=IL+1-I1

WX(II)=X11(IL)

WY(II)=Y11(IL)

END DO

IF (M1.NE.1) THEN

GOTO 5334

END IF

CALL LAG3(XXX,Y2,X11,Y11,WX,WY)

5334     IF (M2.NE.1) THEN

GOTO 5336

END IF

YX=0

5336  RETURN

END

C

SUBROUTINE LAG3(XXX,Y2,X11,Y11,WX,WY)

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

PARAMETER SC=200,PT=200,NNRA=70,NNFN=20,NNKAI=35,NNRD=30

DOUBLE PRECISION NUE,L,KAI,L0,L1,L2,NABLA,KW,KAI1,LTHETA,MGA,KN

&    ,LO,LR,KG,LB1,LB2,LB3,LB2S0,M1,M2,M3,M4,M5,M6,M7,M8

INTEGER BWOSM,TYPE_BK,RDP,CSDMIN,SSBK1,SSBK2

DIMENSION  X11(SC),Y11(SC),WX(SC),WY(SC)

C

Y2=0

DO I3=1, 3

W=1

Z=1

DO J3=1, 3

IF(J3.EQ.I3) THEN

GOTO 5350

END IF

W=W*(XXX-WX(J3))

Z=Z*(WX(I3)-WX(J3))

5350      END DO

Y2=Y2+WY(I3)*W/Z

END DO

RETURN

END

C

C

C      ********** arc cos **********

SUBROUTINE ARC(AA1,ACS)

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

C

IF(ABS(AA1).EQ.1) THEN

ACS=0

GOTO 5361

END IF

ACS=-ATAN(AA1/SQRT(-AA1*AA1+1))+3.14159/2

5361  RETURN

END

C

C

C      ********** hyperbolic tangent **********

SUBROUTINE TANH(TAH,SIT)

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

C

TAH=(EXP(SIT)-EXP(-SIT))/(EXP(SIT)+EXP(-SIT))

RETURN

END

C

C

END of Document